# CMPE2150 Lab 04

Optoisolated TRIAC

Taylor, Moore, and Armstrong

18 Oct 2024

Table of contents

## Optoisolated TRIAC for AC Control

- Using the parts listing for the CNT Year 2 Kit, determine which components U1 and Q1 would have to be for you to build the circuit below:

  - U1:__________

  - Q1: __________

AC Control with Dual TRIACs and Optoisolation

- On the manufacturer’s specification sheet for U1, locate the Typical forward voltage of the LED and, using the current shown for the test condition of the LED’s forward voltage, pick a suitable 10% standard resistor value for R1 (select a value that allows slightly more than required if you have to choose between two): __________ Ω

- Using the manufacturer’s specification sheet for Q1, determine the pinout for Q1. It won’t likely match what you would guess from the schematic symbol!

- Build this circuit far to the right side of your breadboard (to leave space for project components later), using:

  - a normally-open pushbutton switch (PB-NO) for S1,

  - the 24VAC transformer from the CNT Year 2 Kit as V1,

  - the 24V halogen light bulb from the CNT Year 2 Kit,

  - if you don’t have a bench power supply available, use +5VDC from the input of one of your PowerBRICKs or your CNT power supply board for VCC.

- Warning

- Recall that this is directly connected to the 5V supply of your USB port, so make sure your circuit is wired correctly in order to prevent possible damage to your computer.

- Wearing your safety glasses, verify that the lamp lights and goes off when you cycle the switch—don’t leave it on for very long at a time, as it will get hot enough to burn you and melt any plastic nearby, including your breadboard. It will also be quite bright, so try to avoid looking at it for long.

## Build and Test

Once you are satisfied, build your circuit using the parts from your kit and demonstrate it for your instructor during class: __________